Form, Height & Glass:

The Impact of Architectural Design Decisions on Building Energy Performance

Ali Kemal Akbulut  ·  MBL 549E  ·  ITU  Spring 2025–2026

Instructor: Assoc. Prof. Dr. Can Uzun

# 1. The Importance of Data and the Gap It Fills

In architectural practice, energy efficiency decisions are largely based on experience, intuition, or partial simulations that make it impossible to control many variables simultaneously. Designers often cannot answer fundamental questions such as “is glazing or form more decisive?” in an evidence-based manner. This study aims to fill precisely that gap.

The UCI Energy Efficiency dataset, compiled by Tsanas and Xifara (2012), contains 768 Ecotect simulations generated from 12 different building forms, 4 glazing ratios, and 4 orientations. All buildings share the same volume (771.75 m³); only the form, surface area, wall area, roof area, height, orientation, glazing area, and glazing distribution differ. This controlled structure makes it possible to isolate the independent effect of each variable on energy performance. The dataset is openly accessible (CC BY) and was obtained from the UCI ML Repository.

The theoretical significance of this data lies in its direct translation into architectural decision-making. Knowing which parameter dominates energy consumption at the earliest stage of design before form and glazing ratio are settled gives the architect a concrete design lever.

# 2. Data Collection and Management Process

## 2.1 Source and Ethical Considerations

The dataset was obtained as open access from the UCI Machine Learning Repository (ID 242). Its CC BY licence places no restrictions on academic use. As a simulation-based dataset, it involves no human participants and therefore requires no ethics committee approval. Nevertheless, the conditions under which the data were produced climate zone, building materials, and fixed parameters are explicitly documented and presented transparently within the visualisation.

## 2.2 Dataset Structure

The dataset contains eight input variables (X1–X8) and two output variables (Y1: Heating Load, Y2: Cooling Load). X1, relative compactness (range 0.62–0.98), and X7, glazing area ratio (0–40%), are the most critical inputs. Orientation (X6) takes four categorical values (N/E/S/W). Heating load ranges from 6.0 to 49.9 kWh/m²; cooling load from 10.9 to 48.0 kWh/m².

## 2.3 Cleaning and Processing

The dataset arrived complete and consistent, with no missing values or inconsistencies. The only transformation applied in the analysis was averaging the glazing distribution scenarios (X8: uniform distribution + four cardinally biased variants) for visualisation purposes. This makes the primary variable glazing ratio (X7) directly comparable across different building forms. No imputation or normalisation was applied to the original data; raw simulation values are used directly throughout the visualisation.

# 3. Analysis Method

Random Forest (RF) was used to measure the relative effect of each variable on energy performance. RF is a machine learning method that combines the predictions of many decision trees. The importance score assigned to each variable (0–100) reflects that variable’s predictive power within the model.

Results were validated through 10-fold cross-validation repeated 100 times. RF’s mean absolute error (MAE) values are 0.51 kWh/m² for heating load and 1.42 kWh/m² for cooling load. The linear regression method (IRLS) applied for comparison yields 2.14 and 2.21 respectively. Because RF better captures non-linear interactions among architectural variables, it produces approximately 4× lower error than linear regression.

# 4. Visualisation and Findings

The output of this study is a five-tab interactive HTML visualisation: Overview, Parameters, Selected Scenarios, Explorer, and Data & Methods. Using a dark theme with pink and light-blue accent colours, Playfair Display headings, and IBM Plex Mono labels, the visualisation constructs both an analytical and an aesthetic narrative.

Key findings can be summarised as follows:

Glazing ratio is the dominant determinant. Moving from 0% to 40% glazing increases heating load by an average of 78% (14.3 → 25.4 kWh/m²). This effect far exceeds changes in form or height.

Height is significant; building form is secondary. Seven-metre buildings consume on average 2.4× more heating energy than 3.5-metre buildings. Among the 12 forms, the difference at the same height is limited to just 46%.

The effect of orientation is negligible. For the same building facing north, east, south, and west, the heating load difference remains below 0.12 kWh/m²; its RF importance score is 0/100.

The gap between the lowest and highest load is fourfold. B7 (Cubic, 0% glazing, 3.5 m) at 11.9 kWh/m² is the most efficient scenario; B5-40 (Long bar, 40% glazing, 7 m) at 49.7 kWh/m² is the least efficient.

# 5. Conclusion and Limitations

This study systematically compares the impact of architectural design parameters on energy performance and translates those comparisons into a concrete design tool. The visualisation converts abstract RF scores into a platform architects can use directly through an interactive building grid, a slider-based explorer, and scenario showcases.

The main limitation of the study is the simulation origin of the data. Results specific to the Athens climate cannot be transferred directly to other geographies. Because internal gains, HVAC systems, and material properties are held constant, the findings are a measure of the relative effect of formal parameters rather than a prediction of real building energy consumption. Nevertheless, the purpose of the visualisation is not absolute consumption estimation but early-stage guidance for the designer: control glazing area, minimise building height, and treat form decisions as secondary.

# References

Tsanas, A. & Xifara, A. (2012). Accurate quantitative estimation of energy performance of residential buildings using statistical machine learning tools. Energy and Buildings, 49, 560–567.

Breiman, L. (2001). Random forests. Machine Learning, 45(1), 5–32.

UCI Machine Learning Repository. Energy Efficiency Dataset (ID 242). https://archive.ics.uci.edu/dataset/242/energy+efficiency

Kirk, A. (2019). Data Visualisation: A Handbook for Data Driven Design. SAGE Publications.